Security Parancsok

Security K9 license aktiválása PT-ben, ha a router alapból nem támogatja:

Router(config)#license boot module c1900 technology-package securityk9

ezt a parancsot néha 2x kell kiadni egymás után...

go to the global privilege level (enable, config t) and type

- and once the IOS will ask for acceptance type Yes

ACCEPT? [yes/no]: yes

- then jump to privilege using “End” or Ctrl Z

Router# Copy run startup

Save the config using

- and reload the router

Router# reload

- After reload the Security feature is activated

ADMINISTRATIVE ACCESS

R1(config)# security passwords min-length 10

R1(config)# enable algorithm-type scrypt secret cisco12345

R1(config)# line console 0 R1(config-line)# password ciscocon R1(config-line)# exec-timeout 5 0 R1(config-line)# login

R1(config-line)# logging synchronous

R1(config)# line vty 0 4

R1(config-line)# password ciscovtypass R1(config-line)# exec-timeout 5 0 R1(config-line)# transport input telnet R1(config-line)# login

R1(config)# service password-encryption

R1(config)# banner motd $Unauthorized access strictly prohibited!$

R1(config)# username user01 algorithm-type scrypt secret user01pass

R1(config)# username admin privilege 15 algorithm-type scrypt secret cisco12345

SSH

R1(config)# ip domain-name ccnasecurity.com

R1(config)# username admin privilege 15 algorithm-type scrypt secret cisco12345

R1(config)# crypto key zeroize rsa

R1(config)# crypto key generate rsa general-keys modulus 1024

R1(config)# ip ssh version 2

R1(config)# ip ssh time-out 90

R1(config)# ip ssh authentication-retries 2

R1(config)# line vty 0 4

R1(config-line)# privilege level 15 R1(config-line)# login local R1(config-line)# transport input ssh R1(config-line)# exit

Accept only SSH connections:

KECSKEMET(config)# line vty 0 4

KECSKEMET(config-line)# transport input ssh

PCs attached to FastEthernet interfaces:

KECSKEMET(config)# interface FastEthernet0/1

KECSKEMET(config-if)# ip access-group 101 in

KECSKEMET(config)# access-list 101 permit tcp <PC-IP-address> <PC-subnet-mask> any eq 22

KECSKEMET(config)# access-list 101 deny tcp any any eq 22

ADMINISTRATIVE ROLES

R1(config)#enable secret cisco12345 R1(config)#aaa new-model

Belépés “root view”-ba R1# enable view Password: cisco12345 Pl.:

R1(config)# parser view admin1

R1(config-view)# secret admin1pass

R1(config-view)# commands exec include all show

R1(config-view)# commands exec include all config terminal R1(config-view)# commands exec include all debug R1(config-view)# end

Belépés:

R1# enable view admin1 Password: admin1pass R1# show parser view Current view is ‘admin1’

R1(config)#parser view alladminok superview

R1(config-view)# view admin1

R1(config-view)# view admin2

parser view INTERFACE_CONFIG

secret PASSWORD

commands interface

exit

parser view ROUTER_CONFIG

secret PASSWORD

commands router

exit

parser view LINE_CONFIG

secret PASSWORD

commands line vty

exit

username SIMPLE_USER privilege 7 secret PASSWORD

username SUPER_ADMIN privilege 15 secret PASSWORD

NTP

Master

R2# clock set 20:12:00 Dec 17 2014

R2(config)# ntp authentication-key 1 md5 NTPpassword

R2(config)# ntp trusted-key 1 R2(config)# ntp authenticate R2(config)# ntp master 3

Clients

R1(config)# ntp authentication-key 1 md5 NTPpassword

R1(config)# ntp trusted-key 1 R1(config)# ntp authenticate R1(config)# ntp server 10.1.1.2 R1(config)# ntp update-calendar

R1# show ntp associations

R1# debug ntp all

SYSLOG

(Tftpd32 includes a TFTP server)

R1(config)# service timestamps log datetime msec (ajánlott hozzá az ntp server)

R1(config)# logging host 192.168.1.3 (a server IP címe)

Pl.:

Severity Level	Keyword	Meaning

- emergencies	System is unusable

- alerts	Immediate action required

- critical	Critical conditions

- errors	Error conditions

- warnings	Warning conditions

- notifications	Normal but significant condition

- informational	Informational messages

- debugging	Debugging messages

R1(config)# logging trap warnings (ebben az esetnem a 0. 1. 2. 3. 4 szintűeket fogja logolni)

R1# show logging

OSPF authentication

Plain text

R2(config)# interface serial 0/0/1

R2(config-if)# ip ospf authentication

R2(config-if)# ip ospf authentication-key cisco

R3(config)# interface serial 0/0/1

R3(config-if)# ip ospf authentication

R3(config-if)# ip ospf authentication-key cisco

MD5

R1(config)# interface serial 0/0/0

R1(config-if)# ip ospf authentication message-digest

R1(config-if)# ip ospf message-digest-key 1 md5 cisco

R2(config)# interface serial 0/0/0

R2(config-if)# ip ospf authentication message-digest

R2(config-if)# ip ospf message-digest-key 1 md5 cisco

SHA256 Hashing

R1(config)# key chain NetAcad

R1(config-keychain)# key 1

R1(config-keychain-key)# key-string CCNASkeystring

R1(config-keychain-key)#cryptographic-algorithm hmac-sha-256

R1(config)# interface s0/0/0

R1(config-if)# ip ospf authentication key-chain NetAcad

AAA

local

R3(config)# aaa new-model

R3(config)# aaa authentication login default local-case none

R3(config)# aaa authentication login TELNET_LINES local

R3(config)# line vty 0 4

R3(config-line)# login authentication TELNET_LINES

verification

R3(config)# service timestamps debug datetime msec

R3# debug aaa authentication

Server-based

R1(config)# aaa new-model

R1(config)# aaa authentication login default group radius none

R1(config)# aaa accounting exec default start-stop group radius

CEGLED(config)# aaa authentication login TELNET_LINES group radius local CEGLED(config)# line vty 0 4 CEGLED(config-line)# login authentication TELNET_LINES

R1(config)# radius server SERVER-R

R1(config-radius-server)# address ipv4 192.168.1.3 auth-port [port] acct- port [port]

R1(config-radius-server)# key radiuspass

R1(config)# tacacs server SERVER-T

R1(config-radius-server)# address ipv4 192.168.1.3 R1(config-radius-server)# single-connection R1(config-radius-server)# key tacacspass

Régebbi ios-en

R1(config)# radius-server host [IP]

R1(config)# radius-server key [pw]

R1(config)# tacacs-server host [IP] single-connection

R1(config)# tacacs-server key [pw]

ZPF

R3(config)# zone security INSIDE

R3(config)# zone security INTERNET

R3(config)# class-map type inspect match-any INSIDE_PROTOCOLS

R3(config-cmap)# match protocol tcp R3(config-cmap)# match protocol udp R3(config-cmap)# match protocol icmp

R3(config)# policy-map type inspect INSIDE_TO_INTERNET

R3(config-pmap)# class type inspect INSIDE_PROTOCOLS

R3(config-pmap-c)# inspect

R3(config)# zone-pair security INSIDE_TO_INTERNET source INSIDE destination INTERNET

R3(config-sec-zone-pair)# service-policy type inspect INSIDE_TO_INTERNET

R3(config)# interface g0/1

R3(config-if)# zone-member security INSIDE

R3(config)# interface s0/0/1

R3(config-if)# zone-member security INTERNET

R3#show zone-pair security

R3#show policy-map type inspect zone-pair

R3# show zone security

L2 security

The default priority for S1 and S2 is 32769 (32768 + 1 with System ID Extension). Set S1 priority to 0

so that it becomes the root switch.

S1(config)# spanning-tree vlan 1 priority 0

S1# show spanning-tree

Note: You can also use the spanning-tree vlan 1 root primary command to make S1 the root switch for VLAN 1.

S2(config)# interface f0/1

S2(config-if)# switchport mode trunk S2(config-if)# switchport trunk native vlan 99 S1(config-if)# switchport nonegotiate

S1#show interfaces f0/1 switchport

S2(config)# interface f0/18 S2(config-if)# switchport mode access S1(config-if)# spanning-tree portfast

S2(config-if)# spanning-tree bpduguard enable

Note: PortFast and BPDU guard can also be enabled globally with the spanning-tree portfast default and spanning-tree portfast bpduguard commands in global configuration mode.

Note: BPDU guard can be enabled on all access ports that have PortFast enabled. These ports should never receive a BPDU. BPDU guard is best deployed on user-facing ports to prevent rogue switch network extensions by an attacker. If a port is enabled with BPDU guard and receives a BPDU, it is disabled and must be manually re-enabled. An err-disable timeout can be configured on the port so that it can recover automatically after a specified time period.

S1# show spanning-tree interface f0/6 detail

The STP loop guard feature provides additional protection against Layer 2 forwarding loops (STP loops). An STP loop is created when an STP blocking port in a redundant topology erroneously transitions to the forwarding state. This usually happens because one of the ports of a physically redundant topology (not necessarily the STP blocking port) no longer receives STP BPDUs. Having all ports in forwarding state will result in forwarding loops. If a port enabled with loopguard stops hearing BPDUs from the designated port on the segment, it goes into the loop inconsistent state instead of transitioning into forwarding state. Loop inconsistent is basically blocking, and no traffic is forwarded. When the port detects BPDUs again it automatically recovers by moving back into blocking state.

Loop guard should be applied to non-designated ports. Therefore, the global command can be configured on non-root switches.

S2(config)# spanning-tree loopguard default

S1(config-if)# switchport port-security

S1(config-if)# switchport port-security mac-address xxxx.xxxx.xxxx

S2(config-if)# switchport port-security maximum 3 S2(config-if)# switchport port-security violation shutdown S2(config-if)# switchport port-security aging time 120

R1(config)# ip dhcp pool CCNAS

R1(dhcp-config)# network 192.168.1.0 255.255.255.0

R1(dhcp-config)# default-router 192.168.1.1

R1(config)# ip dhcp excluded-address 192.168.1.1 192.168.1.4

Enable DHCP snooping globally.

S1(config)# ip dhcp snooping

S1(config)# ip dhcp snooping information option

Enable DHCP snooping for VLAN 1 and 20.

S1(config)# ip dhcp snooping vlan 1,20

Limit the number of DHCP requests on an interface.

S1(config)# interface f0/6

S1(config-if)# ip dhcp snooping limit rate 10

S1(config-if)# exit

IPSEC 1.

R1(config)# crypto isakmp policy 10 R1(config-isakmp)# hash sha

R1(config-isakmp)# authentication pre-share R1(config-isakmp)# group 14

R1(config-isakmp)# lifetime 3600 R1(config-isakmp)# encryption aes 256 R1(config-isakmp)# end

show crypto isakmp policy

2.

R1(config)# crypto isakmp key cisco123 address 10.2.2.1 3.

R1(config)# crypto ipsec transform-set 50 esp-aes 256 esp-sha-hmac R1(cfg-crypto-trans)#exit

optional:

R1(config)# crypto ipsec security-association lifetime seconds 1800 4.

ACL (érdemleges forgalom)

R1(config)# crypto map CMAP 10 ipsec-isakmp R1(config-crypto-map)# match address 101 R1(config-crypto-map)# set peer 10.2.2.1 R1(config-crypto-map)# set pfs group14 R1(config-crypto-map)# set transform-set 50

R1(config-crypto-map)# set security-association lifetime seconds 900 R1(config-crypto-map)# exit

5.

R1(config)# interface S0/0/0 R1(config-if)# crypto map CMAP

ASA

ASA-Init(config)# hostname CCNAS-ASA

CCNAS-ASA(config)#domain-name ccnasecurity.com CCNAS-ASA(config)# passwd cisco

CCNAS-ASA(config)# enable password class

CCNAS-ASA(config)# clock set 19:09:00 april 192015

CCNAS-ASA(config)# interface vlan 1 CCNAS-ASA(config-if)# nameif inside

CCNAS-ASA(config-if)# ip address 192.168.1.1 255.255.255.0

CCNAS-ASA(config-if)# security-level 100

CCNAS-ASA(config-if)# interface vlan 2 CCNAS-ASA(config-if)# nameif outside

CCNAS-ASA(config-if)# ip address 209.165.200.226 255.255.255.248

CCNAS-ASA(config-if)# no shutdown

CNAS-ASA(config)# interface vlan 3

CCNAS-ASA(config-if)# ip address 192.168.2.1 255.255.255.0 CCNAS-ASA(config-if)# no forward interface vlan 1

CCNAS-ASA(config-if)# nameif dmz

CCNAS-ASA(config-if)# security-level 70 CCNAS-ASA(config-if)# no shut

CCNAS-ASA(config)# interface e0/1

CCNAS-ASA(config-if)#switchport access vlan 1 CCNAS-ASA(config-if)# no shutdown

CCNAS-ASA(config-if)# interface e0/0

CCNAS-ASA(config-if)# switchport access vlan 2 CCNAS-ASA(config-if)# no shutdown

CCNAS-ASA(config-if)#interface Ethernet0/2 CCNAS-ASA(config-if)#switchport access vlan 3 CCNAS-ASA(config-if)#no shut

CCNAS-ASA(config)# show interface ip brief CCNAS-ASA(config)# show switch vlan

CCNAS-ASA(config)# route outside 0.0.0.0 0.0.0.0 209.165.200.225 CCNAS-ASA# show route

NAT

CCNAS-ASA(config)# object network INSIDE-NET

CCNAS-ASA(config-network-object)# subnet 192.168.1.0 255.255.255.0 CCNAS-ASA(config-network-object)# nat (inside,outside) dynamic interface CCNAS-ASA(config-network-object)# end

CCNAS-ASA# show nat

CCNAS-ASA# show xlate

Policy

CCNAS-ASA# class-map inspection_default

CCNAS-ASA(config-cmap)# match default-inspection-traffic

CCNAS-ASA(config)# policy-map global_policy CCNAS-ASA(config-pmap)# class inspection_default CCNAS-ASA(config-pmap-c)# inspect icmp

CCNAS-ASA(config)# service-policy global_policy global CCNAS-ASA(config-pmap-c)# show run policy-map

DHCP

CCNAS-ASA(config)# dhcpd address 192.168.1.5-192.168.1.36 inside CCNAS-ASA(config)# dhcpd dns 209.165.201.2

Note:Other parameters can be specifiedfor clients, such as WINS server, lease length,and domain name.By default,the ASA sets its own IPaddress as the DHCPdefault gateway,so there is no need to configure it. However, to manually configure the default gateway, or set it to a different networking device’s IPaddress,use the following command:

CCNAS-ASA(config)# dhcpd option 3 ip 192.168.1.1 CCNAS-ASA(config)# dhcpd enable inside

AAA - SSH

CCNAS-ASA(config)# username admin password cisco12345 CCNAS-ASA(config)# aaa authentication ssh console LOCAL CCNAS-ASA(config)# crypto key generate rsa modulus 1024 CCNAS-ASA# write mem

CCNAS-ASA(config)# ssh 192.168.1.0 255.255.255.0 inside

CCNAS-ASA(config)# ssh 172.16.3.3 255.255.255.255 outside CCNAS-ASA(config)# ssh timeout 10

PAT

CCNAS-ASA(config)# object network dmz-server CCNAS-ASA(config-network-object)# host 192.168.2.3

CCNAS-ASA(config-network-object)# nat (dmz,outside) static 209.165.200.227

ACL

CCNAS-ASA(config)# access-list OUTSIDE-DMZ permit ip any host 192.168.2.3 CCNAS-ASA(config)# access-group OUTSIDE-DMZ in interface outside

Configure SSH Server on KECSKEMET, BUDAPEST, and SZEGED routers.

- Configure a privileged user for login from the SSH client:

Router(config)# username <username> privilege 15 secret <password>

- Specify privilege level 15 for the VTY lines:

Router(config)# line vty 0 15

Router(config-line)# privilege level 15

- Use local user accounts for mandatory login and validation:

Router(config)# line vty 0 15

Router(config-line)# login local

Router(config-line)# transport input ssh

- Configure SSH timeouts and authentication retries:

Router(config)# ip ssh time-out 60

Router(config)# ip ssh authentication-retries 2

Configure AAA using RADIUS server on BP server. On BUDAPEST and CEGLED routers.

- Configure the authentication service and fallback:

Router(config)# aaa authentication login default group radius local

Router(config)# aaa authentication login no_radius none

Router(config)# aaa authorization exec default group radius if-authenticated

- Create a unique authentication method list for Telnet access on CEGLED router:

Router(config)# aaa authentication login TELNET_LINES group radius local

Router(config)# line vty 0 15

Router(config-line)# login authentication TELNET_LINES

Configure Access List on Szeged router:

Router(config)# access-list 1 permit icmp any any

Router(config)# access-list 2 permit ip any <outside_network_subnet_mask>

Router(config)# access-list 3 permit tcp host <PC4_IP> any eq 22

Router(config)# access-list 4 deny ip any any

Router(config)# interface <interface>

Router(config-if)# ip access-group 1 out

Router(config-if)# ip access-group 2 in

Router(config-if)# ip access-group 3 in

Router(config-if)# ip access-group 4 in

On Obuda router, configure administrative roles.

- Create an interface_config view:

Router(config)# parser view interface_config

Router(config-view)# secret <password>

Router(config-view)# commands configure terminal interface

Router(config-view)# exit

- Create a router_config view:

Router(config)# parser view router_config

Router(config-view)# secret <password>

Router(config-view)# commands configure terminal router

Router(config-view)# exit

- Create a line_config view:

Router(config)# parser view line_config

Router(config-view)# secret <password>

Router(config-view)# commands configure terminal line

Router(config-view)# exit

- Create a simple user with show, debug, and ping commands:

Router(config)# username <username> privilege 3 secret <password>

Router(config)# parser view line_config

Router(config-view)# commands exec include show debug ping

Router(config-view)# exit

- Create a superadmin with all commands:

Router(config)# username <username> privilege 15 secret <password>

Router(config)# parser view line_config

Router(config-view)# commands all

Router(config-view)# exit

Configure BUDAPEST router as a DHCP server and default gateway for VLANs 10 and 20.

- Configure DHCP server:

Router(config)# ip dhcp pool VLAN10

Router(dhcp-config)# network 192.168.10.0 255.255.255.0

Router(dhcp-config)# default-router <BUDAPEST_router_IP>

Router(dhcp-config)# exit

Router(config)# ip dhcp pool VLAN20

Router(dhcp-config)# network 192.168.20.0 255.255.255.0

Router(dhcp-config)# default-router <BUDAPEST_router_IP>

Router(dhcp-config)# exit

- Configure BUDAPEST as the DHCP server for PCs connected to CEGLED router:

Router(config)# interface <interface>

Router(config-if)# ip helper-address <BUDAPEST_router_IP>

Configure Zone-Based Firewall (ZPF) on KECSKEMET router:

Router(config)# zone security inside

Router(config)# zone security outside

Router(config)# zone security dmz

Router(config)# zone-pair security inside-out source inside destination outside

Router(config-sec-zone-pair)# service-policy type inspect INSIDE-OUT

Router(config)# zone-pair security outside-inside source outside destination inside

Router(config-sec-zone-pair)# service-policy type inspect OUTSIDE-INSIDE

Router(config)# zone-pair security dmz-outside source dmz destination outside

Router(config-sec-zone-pair)# service-policy type inspect DMZ-OUTSIDE

Router(config)# zone-pair security inside-dmz source inside destination dmz

Router(config-sec-zone-pair)# service-policy type inspect INSIDE-DMZ

Router(config)# zone-pair security dmz-inside source dmz destination inside

Router(config-sec-zone-pair)# service-policy type inspect DMZ-INSIDE

Router(config)# class-map type inspect match-any HTTP

Router(config-cmap)# match protocol http

Router(config)# class-map type inspect match-any ICMP

Router(config-cmap)# match protocol icmp

Router(config)# class-map type inspect match-any SNMP

Router(config-cmap)# match protocol snmp

Router(config)# policy-map type inspect INSIDE-OUT

Router(config-pmap)# class type inspect HTTP

Router(config-pmap-c)# inspect

Router(config)# policy-map type inspect OUTSIDE-INSIDE

Router(config-pmap)# class type inspect HTTP

Router(config-pmap-c)# inspect

Router(config)# policy-map type inspect DMZ-OUTSIDE

Router(config-pmap)# class type inspect HTTP

Router(config-pmap-c)# inspect

Router(config)# policy-map type inspect INSIDE-DMZ

Router(config-pmap)# class type inspect HTTP

Router(config-pmap-c)# inspect

Router(config)# policy-map type inspect DMZ-INSIDE

Router(config-pmap)# class type inspect HTTP

Router(config-pmap-c)# inspect

Router(config)# interface <interface>

Router(config-if)# zone-member security inside

Router(config-if)# zone-member security outside

Router(config-if)# zone-member security dmz

Configure Layer 2 security:

- Configure Basic Switch Settings:

Switch(config)# hostname <hostname>

Switch(config)# enable secret <password>

- Configure VLAN 1 management interface on all switches:

Switch(config)# interface vlan 1

Switch(config-if)# ip address <ip_address> <subnet_mask>

Switch(config-if)# no shutdown

- Configure SSH Server on S0 and S8:

Switch(config)# ip domain-name <domain_name>

Switch(config)# ip ssh version 2

Switch(config)# crypto key generate rsa

Switch(config)# line vty 0 15

Switch(config-line)# login local

Switch(config-line)# transport input ssh

- Configure a privileged user for login from the SSH client:

Switch(config)# username <username> privilege 15 secret <password>

- Configure S0 as the root switch for VLAN 10, 20, and 30:

Switch(config)# spanning-tree vlan 10 root primary

Switch(config)# spanning-tree vlan 20 root primary

Switch(config)# spanning-tree vlan 30 root primary

- Change the native VLAN for trunk ports on S0, S5, and S6:

Switch(config)# interface <interface>

Switch(config-if)# switchport trunk native vlan <native_vlan>

- Disable DTP on all switch ports:

Switch(config)# interface range <interface_range>

Switch(config-if-range)# switchport nonegotiate

- Protect against STP attack:

Switch(config)# interface range <interface_range>

Switch(config-if-range)# spanning-tree portfast

Switch(config-if-range)# spanning-tree bpduguard enable

- Configure Port Security (1 MAC address, sticky) and Disable Unused Ports:

Switch(config)# interface range <interface_range>

Switch(config-if-range)# switchport mode access

Switch(config-if-range)# switchport port-security

Switch(config-if-range)# switchport port-security maximum 1

Switch(config-if-range)# switchport port-security mac-address sticky

Switch(config-if-range)# shutdown

- Configure DHCP snooping:

Switch(config)# ip dhcp snooping

Switch(config)# interface <interface>

Switch(config-if)# ip dhcp snooping trust

Switch(config)# ip dhcp snooping limit rate <limit_rate>

AAA (Authentication, Authorization, and Accounting):

Configure TACACS+ authorization:

Router(config)# aaa authorization exec default group tacacs+ local

Router(config)# aaa authorization commands 15 default group tacacs+ local

Enable accounting:

Router(config)# aaa accounting exec default start-stop group radius

Router(config)# aaa accounting commands 15 default start-stop group radius

Configure RADIUS accounting:

Router(config)# radius-server accounting host <RADIUS_server_IP>

Router(config)# radius-server accounting key <RADIUS_secret_key>

IPsec (Internet Protocol Security):

Configure IPsec site-to-site VPN:

Router(config)# crypto isakmp profile <profile_name>

Router(config-isakmp-profile)# match identity <remote_peer_identity>

Router(config-isakmp-profile)# keyring <keyring_name>

Router(config)# crypto keyring <keyring_name>

Router(config-keyring)# pre-shared-key address <peer_IP> key <pre shared_key>

Configure IPsec remote access VPN:

Router(config)# crypto isakmp profile <profile_name>

Router(config-isakmp-profile)# match identity user-fqdn <domain_name>

Router(config-isakmp-profile)# client authentication list <authentication_list_name>

Router(config)# crypto ipsec client ezvpn <ezvpn_name>

Router(config-ezvpn)# connect <connect_method>

Router(config-ezvpn)# group <group_name> key <group_key>

Router(config-ezvpn)# mode client

Router(config-ezvpn)# peer <peer_IP>

Router(config-ezvpn)# username <username> password <password>

Zone-Based Policy Firewall (ZPF):

Define inspection rules for specific protocols:

Router(config)# class-map type inspect match-all <class_map_name>

Router(config-cmap)# match protocol <protocol_name>

Apply service policy to interfaces:

Router(config)# interface <interface>

Router(config-if)# service-policy type inspect <policy_name>

ASA (Adaptive Security Appliance):

Enable ASDM access:

ASA(config)# http <inside_network_subnet_mask> <inside_network_interface>

ASA(config)# username <username> password <password> privilege <privilege_level>

ASA(config)# http server enable

ASA(config)# http <management_network_subnet_mask> <management_network_interface>

Configure PAT (Port Address Translation):

ASA(config)# object network <inside_network>

ASA(config-network-object)# nat (inside,<outside/dmz>) dynamic interface

Configure VPN group policy and tunnel group:

ASA(config)# group-policy <group_policy_name> internal

ASA(config-group-policy)# attributes

ASA(config-group-policy)# vpn-tunnel-protocol <protocol>

ASA(config)# tunnel-group <tunnel_group_name> type remote-access

ASA(config)# tunnel-group <tunnel_group_name> general-attributes

ASA(config-tunnel-general)# address-pool <pool_name>

ASA(config)# tunnel-group <tunnel_group_name> ipsec-attributes

ASA(config-tunnel-ipsec)# ikev1 pre-shared-key <pre-shared_key>

ACL (Access Control List):

Extended ACL with additional conditions:

Router(config)# access-list <acl_number> permit/deny <protocol> <source

_address> <source_wildcard> <destination_address> <destination_wildcard> <additional_conditions>

Extended ACL with port/protocol filtering:

Router(config)# access-list <acl_number> permit/deny <protocol> <source_address> <source_wildcard> eq <source_port> <destination_address> <destination_wildcard> eq <destination_port>